2022, C01

1.a)

i)Describe the steps used in a handoff process of a cellular system.

ii) What is the main physical reason for the failure of many MAC schemes known from wired networks? What is

done in wired networks to avoid this effect?

ANS-

(i) Steps in the Handoff Process of a Cellular System

- Measurement of Signal Strength

  - The mobile station continuously measures the signal strength and quality of the serving cell and neighboring cells.

- Reporting to the Base Station

  - These measurements are sent to the serving base station (BS) or BSC/MSC depending on the system.

- Handoff Decision

  - The network compares the received signal strength with thresholds (like RSS, hysteresis, dwell time).

  - If the serving cell becomes weak and a neighboring cell is stronger, a handoff is triggered.

- Resource Allocation in Target Cell

  - The network checks if a free channel is available in the target cell.

  - The MSC/BSC reserves a new channel for the call.

- Execution of Handoff

  - The mobile is instructed to switch to the new frequency/time slot/code of the target cell.

  - The mobile retunes and synchronizes with the new base station.

- Completion and Link Release

  - Once the mobile is connected to the new BS, the old channel is released.

  - Call continues without interruption.

(ii) Main Physical Reason for Failure of Wired MAC Schemes in Wireless Networks

Reason: Hidden Terminal & Signal Attenuation

MAC schemes from wired networks (like CSMA/CD in Ethernet) assume:

- Every node can hear every other node perfectly.

- Collisions can be detected immediately.

But in wireless:

- Nodes may be out of range of each other → hidden terminal problem.

- Signal strength varies due to fading, interference, and attenuation.

- Collision detection is impossible because a node cannot listen while transmitting.

Hence, wired MAC schemes fail physically because wireless links are broadcast, half-duplex, and suffer from hidden/exposed terminals.

What is done in Wired Networks to Avoid This Effect?

Wired networks (like Ethernet) use:

Collision Detection (CD)

- Nodes transmit and listen simultaneously.

- If a collision is detected, they stop transmitting and use binary exponential backoff to retry.

This works because wired channels have:

- Strong, stable signals

- Full physical connectivity

- Ability to detect collisions

In wireless, collision detection is replaced by collision avoidance (e.g., CSMA/CA).

b.

i) Comment on the services provided by 1G, 2G, 3G, 4G, and 5G wireless systems?

ii) 33 MHz of bandwidth is allocated to a cellular system which uses two 25 KHz simplex channels to provide full duplex voice and control channels, compute the number of channels available per cell if the system uses four cell reuse.

ANS-

Comment on the Services Provided by 1G to 5G Wireless Systems

1G (First Generation)

- Provided analog voice communication.

- Supported only voice calls, low capacity.

- No data services, poor security, low quality.

- Examples: AMPS, NMT.

2G (Second Generation)

- Shifted to digital transmission.

- Supported voice calls, SMS, basic data services (up to ~64 kbps).

- Better security and capacity compared to 1G.

- Examples: GSM, CDMA.

2.5G / 2.75G (GPRS/EDGE)

- Introduced packet-switched data, internet browsing.

- Data rates improved to ~100–200 kbps.

3G (Third Generation)

- Provided high-speed mobile data, multimedia support.

- Enabled video calling, mobile TV, internet browsing, email, VoIP.

- Typical speeds 1–2 Mbps (higher in HSPA).

- Examples: WCDMA, CDMA2000.

4G (Fourth Generation)

- Fully IP-based (all-data) system.

- High-speed mobile broadband: 10–100 Mbps.

- Services: HD video streaming, online gaming, VoIP, high-speed internet, mobile hotspots.

- Uses OFDMA and MIMO.

- Example: LTE, LTE-Advanced.

5G (Fifth Generation)

- Ultra-high speed: 1–20 Gbps.

- Ultra-low latency (1 ms).

- Supports massive IoT, autonomous vehicles, AR/VR, smart cities.

- Enhanced mobile broadband, massive machine-type communications, and ultra-reliable low-latency communication.

- Uses mmWave, massive MIMO, beamforming, network slicing.

ii) 33 MHz of bandwidth is allocated to a cellular system which uses two 25 KHz simplex channels to provide full duplex voice and control channels, compute the number of channels available per cell if the system uses four cell reuse.

Solution

Given:

- Total allocated bandwidth = 33 MHz = 33,000 kHz

- Each full-duplex channel requires two 25 kHz simplex channels → 50 kHz

- Frequency reuse factor N = 4

Step 1: Total number of full-duplex channels

Total channels=33,000 kHz50 kHz=660

Step 2: Channels available per cell

Channels per cell=6604=165

✔ Final Answer:

165 channels are available per cell.

CO2

a. ii) Define the role of HLR and VLR in the wireless wide area network (WWAN) architecture.

Role of HLR and VLR in WWAN Architecture

Home Location Register (HLR)

- The HLR is a central database that stores permanent subscriber information.

- It contains details such as mobile number, subscribed services, authentication keys, and the user’s current location area.

- When a user roams or receives a call/SMS, the network checks the HLR to determine where the subscriber is registered.

- It helps in authentication, authorization, and service provisioning for the mobile user.

Visitor Location Register (VLR)

- The VLR is a temporary database that stores information about users who are currently roaming in its area.

- It keeps details needed for call handling: current cell/location area, temporary mobile subscriber identity, and service permissions.

- When a user enters a new MSC/VLR area, the VLR contacts the HLR to fetch the subscriber's information.

- This reduces the need to access the HLR repeatedly, improving speed and efficiency.

b.ii) List three important entities in GSM architecture and explain their functions.

Three Important Entities in GSM Architecture and Their Functions

1. Mobile Switching Centre (MSC)

- Acts as the central controller of the GSM network.

- Responsible for call setup, routing, termination, and handover control.

- Manages communication between mobile users and external networks (PSTN/ISDN).

- Handles mobility functions such as location updating.

2. Base Station Subsystem (BSS)

The BSS consists of BTS + BSC, but you can treat it as a single entity if the question asks for “three.”

- BTS (Base Transceiver Station):
Handles radio transmission and reception with mobile stations. Contains antennas and RF units.

- BSC (Base Station Controller):
Controls multiple BTS units, manages radio resources, performs handoffs within its area, and allocates channels.

3. Home Location Register (HLR)

- A central subscriber database storing permanent user information.

- Contains authentication data, subscribed services, and the user’s last known location area.

- Used for user authentication and call routing.

✔ Short Summary (if needed)

- MSC: Controls calls and mobility.

- BSS (BTS + BSC): Handles radio communication and resource management.

- HLR: Stores permanent subscriber information.

CO4

2022

4.a)

i) What do you mean by neighbor notification address auto-configuration in MIPv6?

Neighbor Notification and Address Auto-Configuration in MIPv6

In Mobile IPv6 (MIPv6), neighbor notification address auto-configuration refers to the process where a mobile node automatically configures its link-local and global IPv6 addresses when it connects to a new network, and then notifies neighboring routers and nodes about its presence.

Key points (write these):

- A mobile node uses IPv6 Stateless Address Auto-Configuration (SLAAC) to create a new address from the network’s prefix, obtained from Router Advertisements (RA).

- After forming the new address, it performs Neighbor Discovery (ND) to check uniqueness (Duplicate Address Detection) and to inform neighbors of the new address.

- This ensures the mobile node becomes reachable on the new network without manual configuration.

ii) Describe a method to achieve soft handoff in MIPv4. How the triangular routing problem is solved in MIPv4?

(ii) Soft Handoff in MIPv4 & Solution to Triangular Routing

1. Method to Achieve Soft Handoff in MIPv4

MIPv4 normally uses hard handoff (break-before-make), but soft handoff can be approximated using:

Foreign Agent (FA) Smooth Handoff

- When a mobile node moves to a new foreign network, both the old FA and the new FA temporarily maintain reachability for the mobile node.

- The old FA forwards any remaining packets to the new FA using the mobile node’s new care-of address (COA).

- Meanwhile, the mobile registers its new COA with the Home Agent (HA), allowing continuous packet delivery.

- This overlap of connectivity creates a soft handoff-like process because packets are not lost during transition.

2. How Triangular Routing Problem is Solved in MIPv4

The Problem:

In basic MIPv4, packets from a Correspondent Node (CN) go to the Home Agent → Foreign Agent → Mobile Node, forming a triangle path which increases delay.

Solution: Route Optimization

- The mobile node sends a binding update to the Correspondent Node.

- The CN stores the mobile's current care-of address in its binding cache.

- After that, the CN sends packets directly to the mobile’s COA, bypassing the HA.

This removes the “triangle” and reduces delay and overhead.

Better understanding - https://youtu.be/nUl_HIw3VLs?si=wqURkvWU84Zq0v5M

NOT IN QUESTION PAPER ADDED BY ME (soft handoff vs hard handoff)

Soft Handoff

- Make-before-break technique.

- Mobile maintains connections with multiple base stations simultaneously during handoff.

- Reduces call drops and improves quality.

- Used in CDMA/WCDMA systems.

Hard Handoff

- Break-before-make technique.

- Mobile disconnects from the old base station first, then connects to the new one.

- Simpler but may cause brief interruption or call drops.

- Used in GSM and other TDMA/FDMA systems.

NOT IN QUESTION PAPER ADDED BY ME,  whats mobility binding?

In Mobile IP (both MIPv4 and MIPv6), a mobility binding is basically a record that links:

Home Address → Care-of Address

Think of it as the network’s way of saying:
“This mobile node’s permanent home address is X, but right now you can reach it at Y.”

More clearly:

- Home Address (HoA): The mobile node’s permanent IP.

- Care-of Address (CoA): The temporary IP it gets in a foreign network.

- Mobility Binding: A mapping stored in the Home Agent (HA) or Correspondent Node (CN) telling them where to forward packets.

iii) When and how is a mobility binding created between the correspondent nodes and mobile nodes? How does mobility binding optimize the route?

When is a Mobility Binding Created?

A mobility binding is created when a mobile node (MN) changes its point of attachment and obtains a new Care-of Address (CoA).
After getting this new CoA, the mobile node sends a Binding Update (BU) to the correspondent node (CN).

How is the Mobility Binding Created?

- The mobile node acquires a new CoA from the visited network.

- It sends a Binding Update message to the CN containing:

  - Home Address (HoA)

  - New Care-of Address (CoA)

- The CN stores this information in its Binding Cache.

- This stored mapping (HoA → CoA) is called mobility binding.

How Does Mobility Binding Optimize the Route?

- After the binding is stored, the CN can send packets directly to the mobile’s CoA, instead of routing everything via the Home Agent (HA).

- This removes the inefficient triangular routing (CN → HA → MN).

- The path becomes CN → MN directly, reducing:

  - Delay

  - Latency

  - Bandwidth wastage

  - Network load

Thus, mobility binding enables route optimization by allowing direct communication with the mobile's current location.

4.b)

i) What do you mean by pre-handoff and post-handoff mechanisms in MIP?

Pre-Handoff and Post-Handoff Mechanisms in Mobile IP (MIP)

1. Pre-Handoff Mechanism

This refers to all actions taken before the mobile node switches from one network to another.

What happens in pre-handoff?

- The mobile node detects that it is leaving the current network (signal becomes weak).

- It starts scanning for new networks and foreign agents.

- It acquires or checks the availability of a new Care-of Address (CoA).

- It prepares for registration to reduce delay.

Simple idea:
“Get everything ready before moving, so the handoff is faster.”

2. Post-Handoff Mechanism

These are the actions done after the mobile node has moved to the new network.

What happens in post-handoff?

- The mobile node sends a Registration Request (Binding Update) to its Home Agent via the new Foreign Agent / new CoA.

- The Home Agent updates its binding table.

- Packet forwarding is switched from the old CoA to the new CoA.

- Buffers or pending packets are transferred (if smooth handoff is used).

Simple idea:
“Once moved, update your records and start receiving packets on the new network.”

ii)Difference Between Care-of Address (CoA) and Co-located Care-of Address

1. Care-of Address (CoA)

- A temporary address assigned to a mobile node when it visits a foreign network.

- Usually provided by a Foreign Agent (FA).

- Multiple mobile nodes can share the same FA-based CoA.

- The FA receives packets on behalf of the mobile node and forwards them.

Key idea:
Address belongs to the Foreign Agent; mobile node borrows it.

2. Co-located Care-of Address

- A unique IP address obtained directly by the mobile node (e.g., via DHCP).

- The mobile node itself acts as the endpoint for the tunnel from the Home Agent.

- Does not require a Foreign Agent — the mobile node handles its own tunneling.

- Offers more independence but requires more resources and IP availability.

Key idea:
Address belongs to the mobile node itself.

Short Difference Table (write this if needed)

| Feature | Care-of Address (FA-based CoA) | Co-located Care-of Address |
| --- | --- | --- |
| Who owns the address? | Foreign Agent | Mobile node |
| Dependency | Requires FA | No FA needed |
| Tunneling endpoint | FA | Mobile node |
| Sharing | Multiple MNs may share | Always unique |

NOT IN PYQ ADDED BY ME

1. Mobile Node (MN)

- The device that moves from one network to another.

- Example: your phone, laptop, or any mobile device using Mobile IP.

Think: “I am the one moving.”

2. Home Network

- The network where the mobile node originally belongs.

- It has a permanent IP address for the mobile node.

Think: “Your hometown.”

3. Home Agent (HA)

- A router in the home network that keeps track of the mobile node.

- When the mobile node is away, the HA forwards packets to its new location.

Think: “Your home post office that forwards your letters when you travel.”

4. Foreign Network

- A network that the mobile node visits when it moves away from its home network.

Think: “A city you are visiting.”

5. Foreign Agent (FA)

- A router in the foreign network.

- Provides the mobile node with a Care-of Address (CoA).

- Receives packets from the HA and delivers them to the mobile node.

Think:
“A temporary post office in the city you are visiting.”

6. Tunnel

- A virtual path used to send packets from the Home Agent to the Mobile Node’s Care-of Address.

- Makes it look like the mobile node is still in the home network.

Think:
“A secret shortcut that forwards your packets from your home to your current location.”

--------------------------------------------------------------------------------------------------------------

What is a Correspondent Node (CN)?

A Correspondent Node is simply any device that is communicating with the mobile node.

It can be:

- A computer

- A server

- Another mobile device

- Any internet host

Basically:
If a device sends data to the mobile node or receives data from it → that device is called the Correspondent Node.

iii)In mobile IPv6, depending on whether or not the correspondent node has knowledge of the mobile’s current care-of-address, its messages destined for the mobile in its current location are sent in different ways.

1.Discuss how messages should be sent to the mobile in its current location.

Case 1: CN Does Not Know the CoA (No Route Optimization)

In this case, the CN only knows the mobile’s Home Address (HoA).

How packets are sent:

- CN sends packets to the Home Address of the mobile node.

- These packets reach the Home Agent (HA) in the home network.

- The HA intercepts them, encapsulates the packets (tunneling).

- HA tunnels them to the mobile’s Care-of Address (CoA).

- The mobile node receives and decapsulates the packets.

Result:
Packets take a longer, indirect path:
CN → HA → Mobile Node
(This is called bidirectional tunneling.)

Case 2: CN Knows the CoA (Route Optimization Active)

Here, the Mobile Node has already sent a Binding Update (BU) to the Correspondent Node, informing it of the CoA.

How packets are sent:

- CN uses the mobility binding (HoA → CoA) stored in its binding cache.

- CN sends packets directly to the CoA of the mobile node.

- The mobile node uses the Home Address Option so the CN still sees the mobile’s home address logically.

Result:
Packets take the shortest path:
CN → Mobile Node directly
(No tunneling through the HA.)

2.IPV6 VS IPV4

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address Length | 32-bit | 128-bit |
| Address Format | Dotted decimal (e.g., 192.168.1.1) | Hexadecimal, colon-separated (e.g., 2001:db8::1) |
| Address Space | ~4.3 billion | ~3.4 × 10³⁸ (huge) |
| Configuration | Manual / DHCP | Auto-configuration (SLAAC) + DHCPv6 |
| Security | IPSec optional | IPSec mandatory |
| Header Type | Variable, complex | Fixed, simplified |
| Broadcast | Supports broadcast | No broadcast (uses multicast) |
| NAT Requirement | Needed (due to shortage) | Not needed |
| Mobility Support | Mobile IPv4 (optional) | Integrated Mobile IPv6 |
| Fragmentation | Done by routers | Done only by sender |

*NAT-network address translation

Advantages of IPv6 Over IPv4

- Larger Address Space
IPv6 uses 128-bit addresses, allowing virtually unlimited devices compared to IPv4’s limited 32-bit space.

- No Need for NAT
Because IPv6 has abundant addresses, Network Address Translation (NAT) is unnecessary, simplifying communication.

- Auto-Configuration (SLAAC)
Devices can configure their own IP addresses without DHCP, making network setup easier and faster.

- Better Security (IPSec Built-in)
IPv6 has mandatory IPSec support, improving authentication, integrity, and data confidentiality.

- Simplified Header
IPv6 uses a fixed-length, simpler header → faster routing and better performance.

- Better Support for Mobility
Mobile IPv6 works more efficiently with built-in mobility support and better route optimization.

- No Broadcast Traffic
IPv6 removes broadcast and uses multicast instead → reduces network congestion.

- Improved QoS Support
Flow label and traffic classes in IPv6 improve handling of real-time traffic like voice/video.

2023, C04

4.a) explain how MIPV6 performs auto configuration and duplicate adddress detection.

1. Auto-Configuration in MIPv6 (Stateless Address Auto-Configuration – SLAAC)

When a mobile node enters a new IPv6 network, it automatically configures a new Care-of Address (CoA) using SLAAC.
The steps are:

Step 1: Receive Router Advertisements (RA)

- The mobile node listens for Router Advertisement (RA) packets from the local router.

- These RAs contain the network prefix of the visited network.

Step 2: Form a New Care-of Address (CoA)

- The mobile node combines:

  - The prefix from the RA

  - Its own interface identifier (often derived from MAC or random value)

- This generates a tentative new IPv6 address, used as its Care-of Address (CoA).

Idea:
The network tells the prefix, the mobile node creates the full address by itself.

2. Duplicate Address Detection (DAD)

After creating a tentative address, the mobile node must check if anyone else in the network is already using it.

Step 1: Send Neighbor Solicitation (NS) Message

- The mobile node sends a Neighbor Solicitation message with the tentative address.

- The message asks:
“Is there any device already using this address?”

Step 2: Wait for Neighbor Advertisement (NA)

- If no one replies, the address is unique → DAD passes.

- If some device responds with a Neighbor Advertisement,
the address is already in use → DAD fails, and the mobile node must generate a new address.

4.b)

when and how is mobility binding created between corresponded nodes and mobile nodes? and how does mobility binding optimize the route?

When and How Mobility Binding Is Created

A mobility binding is created when the Mobile Node (MN) moves to a new network and gets a new Care-of Address (CoA).

How it is created:

- The mobile node enters a new network and obtains a new CoA.

- It sends a Binding Update (BU) message to the Correspondent Node (CN).

- The CN stores the mapping Home Address (HoA) → Care-of Address (CoA) in its binding cache.

- This stored mapping is called the mobility binding.

Meaning:
The CN now knows where the mobile currently is and how to reach it directly.

How Mobility Binding Optimizes the Route

- Without mobility binding, packets follow a long path:
CN → Home Agent → Mobile Node (triangular routing)

- This increases delay and wastes bandwidth.

- With mobility binding:
CN knows the mobile’s CoA →
CN sends packets directly to the Care-of Address, bypassing the Home Agent.

Result:

- Shorter path

- Lower delay

- Less network congestion

- More efficient routing

4.c)

explain with diagrams the paths involved when two mobile hosts communicate using MIPV6 and the correspondent node is in mobile users network.

Scenario

- Mobile Node A (MN-A) is away from its home network → has a Care-of Address (CoA-A).

- Mobile Node B (MN-B) is also away from its home network → also has a Care-of Address (CoA-B).

- MN-B is the Correspondent Node (CN) communicating with MN-A.

In MIPv6, route optimization allows direct communication between the two Care-of Addresses.

Case 1: Before Route Optimization (No Binding Cache at CN)

Communication goes through Home Agents (inefficient).

Diagram

MN-A (CoA-A)                    MN-B (CoA-B)

|                                                               |

[Foreign Network]               [Foreign Network]

|                                                               |

HA-A ---------------------→       HA-B

↑                                                              |

|                                                               ↓

CN = MN-B receives packets via HA-B

Explanation

- MN-A sends packets → HA-A → HA-B → MN-B (CN).

- MN-B sends packets → HA-B → HA-A → MN-A.

This is double triangular routing → slow.

Case 2: After Route Optimization (Binding Updates Exchanged)

Both MN-A and MN-B send Binding Updates (BU) to each other.
Each stores the other's CoA in its binding cache.

Diagram (Optimized Path)

MN-A (CoA-A)  <------------------>  MN-B (CoA-B)

Direct communication (shortest path)

Explanation

- Both nodes now know each other’s current Care-of Address.

- Packets are sent directly CoA-A ↔ CoA-B, without involving Home Agents.

- Home Agents are only used for initial contact.

Detailed Path Flow

1. MN-A → MN-B

MN-A (CoA-A)

|

|      Packet contains:

|   - Source = HoA-A (via Home Address Option)

|   - Destination = CoA-B

|

↓

MN-B (CoA-B)

2. MN-B → MN-A

MN-B (CoA-B)

|

|    Packet contains:

|  - Source = HoA-B

|  - Destination = CoA-A

|

↓

MN-A (CoA-A)

No tunneling needed once optimization is active.

Final Answer Summary (Write This for Exam)

- When two mobile hosts communicate in MIPv6, each mobile node obtains a Care-of Address (CoA) in the visited network.

- Before route optimization, packets pass through both Home Agents, creating triangular routing through HA-A and HA-B.

- After exchanging Binding Updates, both nodes store each other’s CoA in their binding cache.

- Communication then uses direct CoA-to-CoA routing, bypassing all Home Agents.

- The Home Address Option ensures upper layers still see the permanent home address even though packets use the CoA.